The Jiří Myron Theatre

Technical data

Tables depth from the auditorium 3,45m

Proscenium depth from the auditorium to the velvet curtain  5m

Stage depth from the curtain to the black horizon  12,5m

Stage depth from the curtain to the rear iron curtain 13,7m

Rear stage depth from the iron curtain to the wall 8,8m

Proscenium width from wall to wall  20,8m

Stage performance width 11,5m .... total 18m

Rear stage performance width  9,5m .... total 11,7m

Width of gates to decoration warehouse 4m

Width of portals opening  9,8m to 12,9m

Proscenium bottom view height  7,5m

Pockets bottom view height  4,9m

Main stage height to bars  19,5m

Main stage height  to grates 20 m

Main stage bridges height 8,11,14,17 m, railing 1m

Rear stage bridges height 4,6m,railing 1m

Rear stage height to lighting battery  5,2m

Rear stage height to bars 5,6m

Rear stage height to air-conditioning 4m

Height of gates to decoration warehouse 3,9m

Height of gates to decoration warehouse from the courtyard 3,6m

Rear iron stage height 7m

Height of ceiling of decoration warehouse 5,8m

Height of ceiling of decoration warehouse  to lintel 5,4m

Height of booths in decoration warehouse  4m

Portal bridge height – POSITIONS

13 .... 6,90 m  /  12 .... 6,70 m  /  11 .... 6,45 m
                                              10 .... 6,20 m  /   9 .... 5,96 m – golden portal

8 .... 5,70 m  /   7 .... 5,45 m  /    6 .... 5,20 m

5 .... 4,95 m  /   4 .... 4,70 m  /    3 .... 4,46 m

2 .... 4,21 m  /   1 .... 3,96 m  /    0 .... 0,80 m

Manual bars main stage 27 capacity 160kg, bar rod length 12m

+ telescopes 0,5m

Electric bars main stage 3 capacity 320kg,bar length 12m

Point bars main stage 6 capacity 500kg

Point bars on proscenium 6 capacity 160kg

Lighting battery rear stage capacity 60kg

Orchestra pit table vertical shift 0 to - 2,60m, shift period 2,30 min,

- capacity at rest 500 kg/m2

- capacity when activated 0 !

Turntable average  11 m, fluently regulated speed of revs from  0 to 1,9 m/s

capacity at rest 500 kg/m2

capacity when activated 300 kg/m2

Proscenium pre-walls lifting from 0 up to 4,90m capacity 0 kg !!!

Podium

whole          half               square        quarter

200x100      200 x 50       100x100       100x50

_______________________________________________________

17              20                  10                 10                 6

33              27                  10                 10                 5

50              23                  10                 10               10

83                8                                         5

100              9                  10                 10

150              9

200            10                                         5

233              5                                         5

__________________________________________________

board        45                  10                 20                 6

Steel ropes

Ropes average  6,3 mm, loading capacity 350 kg

1m - 18 pc

2m - 18 pc

3m - 18 pc

4m - 18 pc

5m - 18 pc

6m - 18 pc

Ropes average  8 mm, loading capacity 500 kg

1m - 12 pc

2m - 12 pc

3m - 12 pc

4m - 12 pc

5m - 12 pc

6m - 12 pc

Stage technology

| Equipment: | Maximum load: |
| --- | --- |
| Orchestra pit lifts S1 – S2 | 500 kg/m2 static 2 000 kg overall |
| Manual stage bars - single1, 6, 7, 9 – 13 , 15 – 26 | 160 kg |
| Manual stage bars - half –size 2 - 5, 27 – 29 | 160 kg |
| Motorised stage bars M8, M14, M30 | 320 kg |
| Manual stage bars - half –size, rear stageZ1 – Z10 | 130 kg |
| Point bars of forestage C1 – C6 | 100 kg |
| Fire curtain - frontL1 (weight) | 13 700 kg |
| Fire curtain - back L2 (weight) | 5 000 kg |
| Main curtain H1 | 240/90 kg |
| Portal bridge K1 | 3 500/600 kg |
| Lighting battery -stage F1 | 400/100 kg |
| Horizontal lighting battery F2 | 540/160 kg |
| Lighting battery - rear stage F3 | 200/40 kg |
| Prospect lift D1 | 160 kg |
| Blocking wraps – lift light E1 | 200/70 kg |
| Blocking wraps – lift right E2 | 200/70 kg |
| Telescopic hangers of lights left T1 – T4 | 50 kg |
| Telescopic hangers of lights right T5 – T8 | 50 kg |
| Side lift pre-wall left G1 (weight) | 1 540 kg |
| Side lift pre-wall rightG2 | 1 540 kg |
| Turntable - main stage ø 11 m | 300 kg/m2 |